Prelab for Lab6

Xiaoyu Zhou, 1004081147

Part I

2. The resetn signal is a synchronous reset and it is active low. When I do the simulation, I need to begin with setting resetn to be 0.

3. Verilog Code:

// SW[0]:       reset signal

// SW[1]:       input signal (w)

// KEY[0]:      clock

// LEDR[2:0]:   current state

// LEDR[9]:     output (z)

module sequence_detector(SW, KEY, LEDR);

input [9:0] SW;

input [3:0] KEY;

output [9:0] LEDR;

wire w, clock, resetn, z;

reg [2:0] y_Q, Y_D; // y_Q represents current state, Y_D represents next state

localparam A = 3'b000, B = 3'b001, C = 3'b010, D = 3'b011, E = 3'b100, F = 3'b101, G = 3'b110;

// Connect inputs and outputs to internal wires

assign w = SW[1];

assign clock = ~KEY[0];

assign resetn = SW[0];

assign LEDR[9] = z;

assign LEDR[2:0] = y_Q;

// State table

// The state table should only contain the logic for state transitions

// Do not mix in any output logic.  The output logic should be handled separately.

// This will make it easier to read, modify and debug the code.

always @(*)

begin   // Start of state_table

case (y_Q)

A: begin

if (!w) Y_D = A;

else Y_D = B;

end

B: begin

if (!w) Y_D = A;

else Y_D = C;

end

C: begin

if (!w) Y_D = E;

else Y_D = D;

end

D: begin

if(!w) Y_D = E;

else Y_D = F;

end

E: begin

if(!w) Y_D = A;

else Y_D = G;

end

F: begin

if(!w) Y_D = E;

else Y_D = F;

end

G: begin

if(!w) Y_D = A;

else Y_D = C;

end

default: Y_D = A;

endcase

end     // End of state_table

// State Register (i.e., FFs)

always @(posedge clock)

begin   // Start of state_FFs (state register)

if(resetn == 1'b0)

y_Q <= A;

else

y_Q <= Y_D;

end     // End of state_FFs (state register)

// Output logic

// Set z to 1 to turn on LED when in relevant states

assign z = ((y_Q == F) || (y_Q == G));  // To be completed by you!

endmodule

4. Simulation.

Part II

2. Table

3. State

4. Verilog

module control(

input clk,

input resetn,

input go,

output reg  ld_a, ld_b, ld_c, ld_x, ld_r,

output reg  ld_alu_out,

output reg [1:0]  alu_select_a, alu_select_b,

output reg alu_op

);

reg [3:0] current_state, next_state;

localparam  S_LOAD_A        = 4'd0,

S_LOAD_A_WAIT   = 4'd1,

S_LOAD_B        = 4'd2,

S_LOAD_B_WAIT   = 4'd3,

S_LOAD_C        = 4'd4,

S_LOAD_C_WAIT   = 4'd5,

S_LOAD_X        = 4'd6,

S_LOAD_X_WAIT   = 4'd7,

S_CYCLE_0       = 4'd8,

S_CYCLE_1       = 4'd9,

S_CYCLE_2       = 4'd10,

S_CYCLE_3       = 4'd11,

S_CYCLE_4       = 4'd12;

// Next state logic aka our state table

always@(*)

begin: state_table

case (current_state)

S_LOAD_A: next_state = go ? S_LOAD_A_WAIT : S_LOAD_A; // Loop in current state until value is input

S_LOAD_A_WAIT: next_state = go ? S_LOAD_A_WAIT : S_LOAD_B; // Loop in current state until go signal goes low

S_LOAD_B: next_state = go ? S_LOAD_B_WAIT : S_LOAD_B; // Loop in current state until value is input

S_LOAD_B_WAIT: next_state = go ? S_LOAD_B_WAIT : S_LOAD_C; // Loop in current state until go signal goes low

S_LOAD_C: next_state = go ? S_LOAD_C_WAIT : S_LOAD_C; // Loop in current state until value is input

S_LOAD_C_WAIT: next_state = go ? S_LOAD_C_WAIT : S_LOAD_X; // Loop in current state until go signal goes low

S_LOAD_X: next_state = go ? S_LOAD_X_WAIT : S_LOAD_X; // Loop in current state until value is input

S_LOAD_X_WAIT: next_state = go ? S_LOAD_X_WAIT : S_CYCLE_0; // Loop in current state until go signal goes low

S_CYCLE_0: next_state = S_CYCLE_1;

S_CYCLE_1: next_state = S_CYCLE_2;

S_CYCLE_2: next_state = S_CYCLE_3;

S_CYCLE_3: next_state = S_CYCLE_4;

S_CYCLE_4: next_state = S_LOAD_A; // we will be done our two operations, start over after

default:     next_state = S_LOAD_A;

endcase

end // state_table

// Output logic aka all of our datapath control signals

always @(*)

begin: enable_signals

// By default make all our signals 0

ld_alu_out = 1'b0;

ld_a = 1'b0;

ld_b = 1'b0;

ld_c = 1'b0;

ld_x = 1'b0;

ld_r = 1'b0;

alu_select_a = 2'b00;

alu_select_b = 2'b00;

alu_op       = 1'b0;

case (current_state)

S_LOAD_A: begin

ld_a = 1'b1;

end

S_LOAD_B: begin

ld_b = 1'b1;

end

S_LOAD_C: begin

ld_c = 1'b1;

end

S_LOAD_X: begin

ld_x = 1'b1;

end

S_CYCLE_0: begin // Do A <- A * x

ld_alu_out = 1'b1; ld_a = 1'b1; // store result back into A

alu_select_a = 2'b00; // Select register A

alu_select_b = 2'b11; // Also select register A

alu_op = 1'b1; // Do multiply operation

end

S_CYCLE_1: begin // Do A <- Ax * x

ld_alu_out = 1'b1; ld_a = 1'b1; // store result back into A

alu_select_a = 2'b00; // Select register A

alu_select_b = 2'b11; // Also select register A

alu_op = 1'b1; // Do multiply operation

end

S_CYCLE_2: begin // Do B <- B * x

ld_alu_out = 1'b1; ld_b = 1'b1; // store result back into B

alu_select_a = 2'b01; // Select register B

alu_select_b = 2'b11; // Also select register x

alu_op = 1'b1; // Do multiply operation

end

S_CYCLE_3: begin // Do A <- Axx + Bx

ld_alu_out = 1'b1; ld_a = 1'b1; // store result back into A

alu_select_a = 2'b00; // Select register A

alu_select_b = 2'b01; // Also select register B

alu_op = 1'b0; // Do multiply operation

end

S_CYCLE_4: begin

ld_r = 1'b1; // store result in result register

alu_select_a = 2'b00; // Select register A

alu_select_b = 2'b10; // Select register C

alu_op = 1'b0; // Do Add operation

end

// default:    // don't need default since we already made sure all of our outputs were assigned a value at the start of the always block

endcase

end // enable_signals

// current_state registers

always@(posedge clk)

begin: state_FFs

if(!resetn)

current_state <= S_LOAD_A;

else

current_state <= next_state;

end // state_FFS

endmodule

5.

6. Simulation